PelvynTM Robotics Marine Robotics Division — Embedded Systems Laboratory ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

FINAL PROJECT  —  STAGE 2

Real-Time Embedded Telemetry

& Health Monitoring Node

for Marine Robotics Applications

Author: K. Prajith Raj Organisation: PelvynTM Robotics — Marine Robotics Division Institution: St. Joseph's Institute of Technology Department: Electronics & Communication Engineering (ECE) Document Type: Complete Technical Documentation — Final Stage 2 Report

CONFIDENTIAL ¦ PelvynTM Robotics ¦ Marine Robotics Division ¦ 2025

Table of Contents

1.  System Architecture Diagram  ···  2

2.  Wiring Diagram & Pin Configuration  ···  3

3.  Software Architecture Diagram  ···  4

4.  Firmware Flowchart  ···  4

5.  Project Folder Structure  ···  5

6.  GitHub Repository Documentation  ···  5

7.  MQTT Architecture & Cloud Design  ···  6

8.  Technical Report (3-Page Summary)  ···  7

9.  Video Demonstration Script  ···  9

1.  System Architecture Diagram

The proposed system uses an ESP32 controller as the central processing unit. Sensor data from the MPU6050 IMU, BMP280 pressure sensor, and DS18B20 temperature sensor are periodically acquired and processed at 1 Hz. The collected data is stored locally on an SD card in structured CSV format and simultaneously transmitted through MQTT over WiFi to a cloud dashboard for remote real-time monitoring and alerting.

Figure 1: Complete System Architecture & Data Flow Diagram

| MPU6050 IMU (Accel + Gyro) I2C — GPIO 21/22 |  | BMP280 Pressure & Altitude I2C — GPIO 21/22 |  |  |  | DS18B20 Waterproof Temperature 1-Wire — GPIO4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ▼ |  |  | ▼ |  |  |  | ▼ |
| ESP32 DevKit V1 — Central Processing Unit Dual-core 240 MHz · 520 KB SRAM · Wi-Fi 802.11b/g/n · Bluetooth 4.2 GPIO ¦ UART ¦ SPI ¦ I2C ¦ ADC ¦ PWM |  |  |  |  |  |  |  |
| ▼ |  |  |  |  | ▼ |  |  |  |
| Micro SD Card Module Local CSV Data Logging SPI — GPIO 5/18/19/23 |  |  |  |  | WiFi MQTT Client Telemetry to Cloud Dashboard broker.hivemq.com ¦ pelvyn/# |  |  |
|  |  |  |  |  | ▼ |  |  |  |
|  |  |  |  |  | MQTT Broker broker.hivemq.com. ▼. Cloud Dashboard Real-time monitoring + Alerts. ▼. Field Operator / User Alerts + Data Access. |

Data flows: Sensors → ESP32 → [SD Card | WiFi → MQTT → Cloud Dashboard → User]

System Architecture Summary • 3 sensors communicate to ESP32 over I2C (MPU6050, BMP280) and 1-Wire (DS18B20). • ESP32 logs all readings to Micro SD Card (SPI) every second — zero data loss offline. • WiFi MQTT client publishes JSON telemetry to broker.hivemq.com on pelvyn/telemetry. • Cloud dashboard visualises live data; alerts fire when thresholds are breached. • Store-and-forward: if WiFi drops, SD card buffers data and syncs on reconnection.

2.  Wiring Diagram & Pin Configuration

All three sensors and the SD card module connect to the ESP32 DevKit V1 using standard embedded interfaces. The MPU6050 and BMP280 share the same I2C bus (GPIO21 SDA, GPIO22 SCL) — each uses a distinct 7-bit I2C address (0x68 and 0x76 respectively) so no bus conflicts occur. The DS18B20 uses the single-wire 1-Wire protocol on GPIO4 with a 4.7 kΩ pull-up resistor. The SD card uses the ESP32 hardware SPI peripheral.

Figure 2: Complete Wiring & Pin Assignment Table

| MPU6050 → ESP32 MPU6050; ESP32 Pin. VCC; 3.3 V. GND; GND. SDA; GPIO 21. SCL; GPIO 22. INT; GPIO 35 (optional). |  | BMP280 → ESP32 BMP280; ESP32 Pin. VCC; 3.3 V. GND; GND. SDA; GPIO 21. SCL; GPIO 22. CSB; 3.3 V (I2C). |  | DS18B20 → ESP32 DS18B20; ESP32 Pin. VCC (Red); 3.3 V. GND (Black); GND. DATA (Yellow); GPIO 4. Pull-up; 4.7 kΩ to 3.3V. |  | SD Module → ESP32 SD Module; ESP32 Pin. VCC; 3.3 V. GND; GND. CS; GPIO 5. MOSI; GPIO 23. MISO; GPIO 19. SCK; GPIO 18. |
| --- | --- | --- | --- | --- | --- | --- |

Wiring Notes & Best Practices • All sensors powered at 3.3 V — do NOT connect to 5 V (ESP32 GPIO is 3.3 V logic). • I2C bus: MPU6050 (address 0x68) and BMP280 (address 0x76) share GPIO21/22 without conflict. • DS18B20: mandatory 4.7 kΩ pull-up resistor between DATA and VCC for reliable communication. • SD Card: use SPI Mode 0; ensure common GND between ESP32 and all modules. • Decoupling: place 100 nF ceramic capacitor across VCC/GND of each sensor for noise filtering.

3.  Software Architecture Diagram

| Figure 3: Layered Firmware Architecture APPLICATION LAYER Main execution loop — orchestrates all modules. ▼. Sensor Manager readIMU() · readPressure() · readTemperature(). ▼. SD Logger CSV write · file flush · circular buffer management. ▼. MQTT Client WiFi connect · broker connect · topic publish. ▼. Health Monitor Sensor status · SD status · WiFi status · uptime. ▼. Status Manager Alarm flags · LED indicators · Serial debug print. ▼. ESP32 HARDWARE ABSTRACTION LAYER (HAL) Arduino / ESP-IDF drivers — GPIO · I2C · SPI · UART · WiFi stack. | Module Descriptions Sensor Manager - Initialises all three sensor peripherals on boot. - Provides readIMU(), readPressure(), readTemperature() APIs. - Returns structured SensorData struct to Application Layer. SD Logger - Mounts FAT filesystem on SD card at boot. - Writes CSV rows: timestamp, temp, pressure, accX, accY, accZ, status. - Calls file.flush() after every write to prevent data loss. MQTT Client - Manages WiFi connection with auto-reconnect on drop. - Publishes JSON payload to pelvyn/telemetry every second. - Publishes alarm flags to pelvyn/alerts on threshold breach. Health Monitor - Polls sensor driver status registers every 5 seconds. - Monitors WiFi RSSI and SD card mount status. - Reports uptime, loop cycle time, and free heap memory. Status Manager - Maintains global alarm flag state machine. - Drives status LEDs (green = normal, red = alarm). - Outputs human-readable debug messages on Serial @ 115200 baud. |
| --- | --- |

4.  Firmware Execution Flowchart

| Figure 4: Firmware Execution Flow START / POWER ON Boot ESP32 firmware. ▼. Initialise Hardware GPIO · I2C · SPI · UART · WiFi stack. ▼. Initialise Sensors MPU6050 · BMP280 · DS18B20. ▼. Initialise SD Card Mount filesystem · create log.csv. ▼. Connect to WiFi SSID + password · DHCP → IP assigned. ▼. Connect to MQTT Broker broker.hivemq.com : 1883. ▼. Read All Sensors IMU · Pressure · Temperature · Leak. ▼. Store Data to SD Card Append timestamped CSV row · flush. ▼. Publish MQTT Packet pelvyn/telemetry · pelvyn/health. ▼. Check System Health Sensor OK? · SD OK? · WiFi OK?. ▼. Generate Warnings Flag alarms · Serial print · LED. ▼. DELAY 1 s → REPEAT loop() — infinite monitoring cycle. | Execution Flow Notes The firmware follows a deterministic initialisation sequence on power-up, followed by an infinite monitoring loop executing at a 1-second interval. Initialisation Phase (setup()) - Serial port initialised at 115200 baud for debug output. - I2C bus started; MPU6050 and BMP280 polled — error printed if absent. - 1-Wire bus initialised; DS18B20 address scanned and stored. - SPI bus started; SD card mounted; log.csv header written. - WiFi credentials loaded from config.h; connection attempted with 30 s timeout. - MQTT client configured with broker IP, port, and client ID. Main Loop Phase (loop()) - Sensor Manager reads all three sensors in sequence. - SD Logger appends one CSV row with millis() timestamp. - MQTT Client publishes JSON telemetry packet. - Health Monitor checks sensor/SD/WiFi status flags. - Status Manager evaluates alarms and updates LED + alerts topic. - delay(1000) enforces 1 Hz sampling rate before repeating. Alarm Conditions Parameter; Threshold; Alarm Flag. Temperature; > 60 °C; HIGH_TEMP. Pressure; > 1100 hPa; HIGH_PRESSURE. Vibration; ΔAcc > 2.5 g; STRUCTURE_FAULT. WiFi RSSI; < -80 dBm; WEAK_SIGNAL. SD Card; Mount fail; SD_ERROR. |
| --- | --- |

5.  Project Folder Structure

The repository follows a professional PlatformIO-compatible directory layout separating source code, documentation, diagrams, and assets into clearly defined directories. This structure enables immediate cloning and compilation without manual file reorganisation.

| Directory Tree Pelvyn_Stage2_Project/ ├── src/ │ ├── main.cpp │ ├── sensor_manager.cpp │ ├── sd_logger.cpp │ ├── mqtt_client.cpp │ └── health_monitor.cpp ├── include/ │ ├── config.h │ └── sensor_types.h ├── diagrams/ │ ├── system_architecture.png │ ├── wiring_diagram.png │ ├── software_architecture.png │ ├── flowchart.png │ └── mqtt_architecture.png ├── report/ │ └── technical_report.pdf ├── images/ │ ├── setup.jpg │ └── testing.jpg ├── data/ │ └── sample_log.csv ├── platformio.ini ├── README.md └── LICENSE. | Directory Purpose Directory / File; Purpose. src/; All C++ firmware source files — one file per module for clarity. include/; Shared headers — config.h holds WiFi/MQTT credentials and pin definitions. diagrams/; All architecture and wiring diagrams in PNG format. report/; PDF technical report for submission. images/; Hardware setup and testing photos for README. data/; Sample CSV output from a real logging session. platformio.ini; PlatformIO build config — board, framework, libraries. README.md; Full project documentation for GitHub. LICENSE; MIT licence — open source. |
| --- | --- |

6.  GitHub Repository Documentation (README.md)

| Project Overview Real-Time Embedded Telemetry & Health Monitoring Node for Marine Robotics — an ESP32-based IoT system that continuously reads IMU, pressure, and temperature sensors, logs data to SD card, and publishes MQTT telemetry to a cloud dashboard. Hardware Components Component; Model; Interface. Microcontroller; ESP32 DevKit V1; —. IMU; MPU6050; I2C 0x68. Pressure Sensor; BMP280; I2C 0x76. Temperature; DS18B20 Waterproof; 1-Wire GPIO4. Storage; Micro SD Module; SPI GPIO5/18/19/23. Communication; Built-in WiFi; 802.11b/g/n. Key Features - 1 Hz real-time multi-sensor data acquisition - Local CSV data logging with millisecond timestamps - MQTT telemetry over WiFi to cloud dashboard - 5-module health monitoring with auto-reconnect - JSON payload with temperature, pressure, IMU, status - Store-and-forward: SD buffer when WiFi unavailable - Serial debug output at 115200 baud | Installation & Setup - Install Arduino IDE 2.x or PlatformIO in VS Code. - Add ESP32 board: Tools → Board Manager → search 'esp32' → install Espressif. - Install libraries: PubSubClient, Adafruit BMP280, DallasTemperature, OneWire, MPU6050. - Open include/config.h and set WIFI_SSID, WIFI_PASSWORD, MQTT_BROKER. - Select board: ESP32 Dev Module. Flash speed: 921600. - Upload main.cpp. Open Serial Monitor at 115200 baud. MQTT Configuration Setting; Value. Broker; broker.hivemq.com. Port; 1883. Client ID; pelvyn-node-01. Telemetry; pelvyn/telemetry. Health; pelvyn/health. Alerts; pelvyn/alerts. Running the Project - Power the ESP32 via USB or 5V barrel jack. - Serial Monitor shows [INIT] status for each module. - Sensor readings print every second: T=28.5°C P=1013hPa. - SD card creates /log.csv — verify with card reader. - Subscribe to pelvyn/# on any MQTT client to see live data. |
| --- | --- |

7.  MQTT Architecture & Cloud Communication Design

| Figure 5: MQTT End-to-End Communication Stack ESP32 Node WiFi 802.11b/g/n client. ▼ WiFi / TCP / MQTT ▼. WiFi Access Point IEEE 802.11 → TCP/IP network. ▼ WiFi / TCP / MQTT ▼. MQTT Broker broker.hivemq.com : Port 1883. ▼ WiFi / TCP / MQTT ▼. Cloud Dashboard Node-RED / Grafana / ThingsBoard. ▼ WiFi / TCP / MQTT ▼. Field Operator Browser / Mobile alert interface. MQTT Topic Architecture Topic; Direction; Content. pelvyn/telemetry; PUBLISH; Full sensor JSON payload (1 Hz). pelvyn/health; PUBLISH; System status, uptime, WiFi RSSI. pelvyn/alerts; PUBLISH; Alarm flags when threshold breached. pelvyn/config; SUBSCRIBE; Remote config updates from dashboard. | JSON Telemetry Payload { "device_id": "pelvyn-node-01", "timestamp": 1700000000000, "temperature": 28.5, "pressure": 1013.25, "altitude": 8.4, "accX": 0.12, "accY": 0.08, "accZ": 9.81, "gyroX": 0.02, "gyroY": -0.01, "gyroZ": 0.00, "status": "NORMAL", "sd_ok": true, "wifi_rssi": -62, "uptime_s": 3600 }. Health Payload { "mpu6050_ok": true, "bmp280_ok": true, "ds18b20_ok": true, "sd_card_ok": true, "wifi_ok": true, "free_heap": 187432, "loop_ms": 12 }. |
| --- | --- |

8.  Technical Report

TECHNICAL REPORT — PAGE 1 OF 3 Introduction & Hardware Selection

8.1  Introduction

The Real-Time Embedded Telemetry and Health Monitoring Node is a compact, autonomous IoT device developed by PelvynTM Robotics for continuous monitoring of marine robotic platforms. Operating in demanding underwater environments, such systems must deliver reliable, low-latency sensor data acquisition, robust local data retention, and seamless cloud connectivity to enable remote situational awareness and predictive maintenance.

This project delivers a complete embedded solution on the ESP32 microcontroller, integrating three complementary sensor modalities — inertial measurement (MPU6050), ambient pressure/altitude (BMP280), and water temperature (DS18B20) — with a dual-path data pipeline: local SD card CSV logging and real-time MQTT telemetry over WiFi. A dedicated health monitor subsystem continuously verifies the operational status of all peripherals and the communication link, generating prioritised alert flags when anomalous conditions are detected.

8.2  Hardware Selection Justification

| Component | Model Selected | Key Justification |
| --- | --- | --- |
| Main Controller | ESP32 DevKit V1 | Dual-core 240 MHz; native WiFi + BT; rich I2C/SPI/UART peripherals; Arduino ecosystem; deep-sleep for power saving; ample GPIO (34 pins). |
| IMU | MPU6050 | 6-DOF in single chip (3-axis accel + 3-axis gyro); I2C 0x68; DMP hardware fusion; low power; essential for structural vibration and orientation monitoring. |
| Pressure Sensor | BMP280 | High-resolution barometric pressure (±1 hPa) and temperature; I2C 0x76; shares bus with MPU6050; compact, low-power, wide supply range 1.8–3.6 V. |
| Temperature | DS18B20 | IP68 waterproof probe; ±0.5 °C accuracy over -55 to +125 °C; 1-Wire protocol uses only one GPIO pin; supports multiple sensors on single bus. |
| Local Storage | Micro SD Module | SPI interface; FAT32 filesystem; up to 32 GB; non-volatile — retains all data through power cycles and communication blackouts; industry-standard format. |
| Communication | ESP32 Built-in WiFi | No external module needed; MQTT over TCP/IP; 802.11b/g/n; WPA2 security; RSSI monitoring for link quality assessment. |

TECHNICAL REPORT — PAGE 2 OF 3 Software Architecture, Communication & Data Logging

8.3  Software Architecture & Firmware Modules

The firmware is organised as a six-module layered architecture running on the Arduino/ESP-IDF framework. Each module has a clearly defined single responsibility, communicating through structured data types rather than global variables, which enables independent testing and future replacement.

| Module | Responsibilities & Implementation Details |
| --- | --- |
| Sensor Manager | Initialises MPU6050 (I2C 0x68), BMP280 (I2C 0x76), and DS18B20 (1-Wire GPIO4) on boot. Exposes readIMU(), readPressure(), readTemperature() functions returning typed structs. Handles re-initialisation on sensor fault detection. |
| SD Logger | Mounts FAT32 filesystem via SPI. Creates /log.csv with header row on first boot. Appends one record per loop iteration with millis() timestamp. Calls flush() after each write to prevent data loss on unexpected power failure. Implements circular buffer for high-frequency bursts. |
| MQTT Client | Uses PubSubClient library over ESP32 WiFi stack. Manages connection lifecycle: initial connect, keepalive PING, auto-reconnect within 5 seconds of drop. Serialises sensor data into compact JSON using ArduinoJson. Publishes to pelvyn/telemetry (1 Hz), pelvyn/health (0.2 Hz), pelvyn/alerts (on-event). |
| Health Monitor | Polls sensor driver status every 5 iterations. Tracks WiFi RSSI — flags WEAK_SIGNAL if < -80 dBm. Monitors SD card mount state. Reports free heap memory and loop execution time to detect resource exhaustion. Stores status in a HealthStatus struct shared with Status Manager. |
| Status Manager | Evaluates alarm conditions against configurable thresholds. Maintains a priority-ordered alarm flag (LEAK > HIGH_TEMP > HIGH_PRESSURE > STRUCTURE_FAULT). Drives GPIO-connected status LEDs. Prints formatted debug output to Serial at 115200 baud for real-time monitoring via USB. |
| App Layer (main) | Calls setup() once for sequential hardware initialisation. loop() orchestrates all five modules in order: read → log → publish → health check → status update → delay(1000). All timing is non-blocking where possible to avoid sensor read latency buildup over multi-hour missions. |

8.4  Communication Workflow

On each 1-second loop iteration, the MQTT Client serialises sensor readings into a JSON object and calls client.publish("pelvyn/telemetry", payload). If the broker connection is lost, PubSubClient's reconnect() is called; failed reconnections are retried with 5-second back-off. During WiFi outages, the SD Logger continues buffering data locally. When connectivity is restored, cached records are uploaded in chronological order to prevent data gaps in the cloud dashboard.

The MQTT broker (broker.hivemq.com) routes messages to subscribed dashboard clients. ThingsBoard or Node-RED dashboards visualise temperature trends, pressure history, IMU orientation, and system health. Alert rules fire push notifications or emails when alarm flags appear on pelvyn/alerts.

8.5  Data Logging — CSV Format

SD Card CSV Log — Sample Output (log.csv) Time_ms,Temp_C,Pressure_hPa,Altitude_m,AccX_g,AccY_g,AccZ_g,GyroX,GyroY,GyroZ,Status 1000,28.3,1013.20,8.2,0.02,0.01,9.81,0.00,0.00,0.01,NORMAL 2001,28.4,1013.21,8.1,0.02,0.01,9.81,0.00,0.01,0.00,NORMAL 3002,28.5,1013.19,8.3,0.03,0.02,9.82,0.01,0.00,0.01,NORMAL 4003,61.2,1013.18,8.4,0.02,0.01,9.81,0.00,0.00,0.00,HIGH_TEMP 5004,62.0,1115.50,8.5,0.02,0.01,9.82,0.00,0.00,0.01,HIGH_PRESSURE

TECHNICAL REPORT — PAGE 3 OF 3 Challenges, Future Improvements & Conclusion

8.6  Engineering Challenges & Solutions

| Challenge | Root Cause | Solution Implemented |
| --- | --- | --- |
| WiFi Reliability | ESP32 WiFi stack disconnects on DHCP lease renewal or router idle timeout. | PubSubClient auto-reconnect with 5 s back-off; WiFi.setAutoReconnect(true); RSSI monitoring and WEAK_SIGNAL flag to pre-warn of impending dropout. |
| Sensor Calibration Drift | BMP280 pressure baseline shifts with ambient temperature; MPU6050 gyro bias accumulates. | BMP280: sea-level pressure set in config.h and re-applied after reboot. MPU6050: offset calibration run in initIMU() using 500-sample averaging routine; offsets stored in flash. |
| SD Card Sync & Corruption | FAT filesystem corruption on sudden power loss if write buffer not flushed. | file.flush() called after every single write operation; watchdog timer resets ESP32 cleanly on hang to prevent partial writes; verified with 24-hour power-cycle stress test. |
| I2C Bus Contention | MPU6050 and BMP280 share I2C bus; simultaneous read attempts cause NACKs. | Sensor Manager reads sensors sequentially, never concurrently; 50 µs inter-transaction delay inserted; Wire.setTimeout(1000) prevents indefinite I2C hang. |
| DS18B20 Parasitic Power | Waterproof probe wiring length increases capacitance, causing read errors. | External 4.7 kΩ pull-up confirmed; parasite power mode disabled (VCC mode); 750 ms conversion wait enforced; up to 3 retry attempts on CRC mismatch. |
| MQTT Packet Loss | Broker drops QoS 0 packets during network congestion. | Critical alarm messages upgraded to QoS 1 (at-least-once delivery) via PubSubClient publish overload; normal telemetry remains QoS 0 to minimise latency. |

8.7  Future Improvements

| Near-Term (Phase 2) - AI Anomaly Detection: deploy Isolation Forest model (sklearn) on Raspberry Pi edge node — detect pipeline leaks and structural faults before threshold breach. - OTA Firmware Updates: implement ArduinoOTA over WiFi to update firmware without physical access — critical for deployed subsea nodes. - Battery Monitoring: add INA219 current sensor to monitor battery voltage, current, and state-of-charge; publish to pelvyn/power MQTT topic. - Leak Detection Sensor: integrate conductive probe leak sensor for direct water ingress detection into the electronics housing. | Long-Term (Phase 3+) - Cloud Analytics: ingest MQTT data into AWS IoT Core → S3 → QuickSight for historical trending, predictive maintenance reports, and fleet management. - TensorFlow Lite Inference: run quantised LSTM anomaly model directly on ESP32 — eliminates cloud dependency for real-time fault classification. - Mesh Networking: deploy multiple nodes communicating via ESP-NOW mesh — enables monitoring of long pipeline sections with a single surface gateway. - Acoustic Integration: replace WiFi with acoustic modem for deep-water deployments beyond WiFi range — implementing full Task 3 architecture. |
| --- | --- |

8.8  Conclusion

The PelvynTM Real-Time Embedded Telemetry & Health Monitoring Node successfully demonstrates a complete embedded IoT pipeline for marine robotics applications. The system acquires multi-modal sensor data at 1 Hz, maintains zero-loss local logging on SD card, and delivers live MQTT telemetry to a cloud dashboard over WiFi. The modular firmware architecture ensures maintainability and extensibility, with clear pathways to AI integration, OTA updates, and acoustic communication for deeper deployments.

The project fulfils all Stage 2 deliverables: system and wiring diagrams, layered software architecture, firmware flowchart, professional project structure, complete GitHub documentation, MQTT cloud design, full technical report, and a structured video demonstration script — establishing a solid engineering foundation for PelvynTM Robotics' commercial underwater inspection programme.

9.  Video Demonstration Script (5 Minutes)

The following is a structured script for a 5-minute professional video demonstration of the complete system. Each minute has a specific focus area and suggested visuals.

| Minute | Script & Visuals |
| --- | --- |
| 0:00–1:00 Introduction | VISUAL: Title card — "PelvynTM Robotics — Real-Time Marine Telemetry Node". SCRIPT: "Hello, I'm K. Prajith Raj from the Electronics and Communication Engineering department at St. Joseph's Institute of Technology, presenting the PelvynTM Robotics Stage 2 Final Project — a Real-Time Embedded Telemetry and Health Monitoring Node for Marine Robotics Applications. This system continuously monitors temperature, pressure, and structural vibration, logs all data locally to an SD card, and transmits live telemetry to a cloud dashboard via MQTT over WiFi. Let me walk you through the complete system." |
| 1:00–2:00 Hardware Explanation | VISUAL: Slowly pan over assembled hardware — ESP32 board, all three sensors, SD module, USB power. SCRIPT: "Here is our hardware setup. The brain of the system is the ESP32 DevKit V1 — a dual-core processor running at 240 MHz with built-in WiFi. Connected via I2C are the MPU6050 six-axis IMU for acceleration and gyroscope readings, and the BMP280 pressure and altitude sensor — both sharing GPIO pins 21 and 22. The DS18B20 waterproof temperature probe connects via the 1-Wire protocol on GPIO4 with a 4.7 kΩ pull-up resistor. The SD card module connects via the SPI bus on GPIO 5, 18, 19, and 23. All powered at 3.3 volts." |
| 2:00–3:00 Live Sensor Readings | VISUAL: Screen capture of Arduino IDE Serial Monitor scrolling with live data. SCRIPT: "Let me show the system running. In the Serial Monitor at 115200 baud, you can see the initialisation sequence — all sensors confirmed OK. Now live readings scroll every second: Temperature 28.5 degrees Celsius, Pressure 1013 hPa, Altitude 8.4 metres, Acceleration X 0.02g, Y 0.01g, Z 9.81g — confirming the sensor is stationary and level. Status is NORMAL. I'll now touch the DS18B20 probe to trigger a temperature alarm — watch the status change to HIGH_TEMP as the reading exceeds 60 degrees Celsius." |
| 3:00–4:00 SD Card Logging | VISUAL: Remove SD card, insert into laptop, open log.csv in Excel/text editor showing timestamped rows. SCRIPT: "Every sensor reading is simultaneously written to the Micro SD card. I've removed the card after 5 minutes of operation. Opening log.csv, you can see each row contains the timestamp in milliseconds, temperature, pressure, altitude, all three acceleration axes, all three gyroscope axes, and the alarm status. This data persists even without WiFi connectivity — the system never loses a reading. The file size grows at approximately 500 bytes per second of operation, allowing days of continuous logging on a standard 32 GB card." |
| 4:00–5:00 MQTT Dashboard & Conclusion | VISUAL: Browser window showing MQTT dashboard (MQTT Explorer or HiveMQ web client) with live values updating. Then show system architecture diagram. SCRIPT: "Simultaneously, the ESP32 publishes live data to the HiveMQ public broker on the pelvyn/telemetry topic. Here in the MQTT dashboard you can see temperature, pressure, and IMU values updating every second in real time. The pelvyn/health topic shows system status — all sensors OK, WiFi RSSI -62 dBm, uptime 5 minutes. The pelvyn/alerts topic shows the HIGH_TEMP alarm we triggered earlier. This complete system — multi-sensor acquisition, local logging, and cloud MQTT telemetry — demonstrates the foundation for PelvynTM Robotics' commercial underwater inspection platform. Thank you." |

| Prepared By K. Prajith Raj B.E. Electronics & Communication Engineering St. Joseph's Institute of Technology PelvynTM Robotics — Marine Robotics Division | Document Details Stage: Final Project — Stage 2 Complete S |
| --- | --- |